DT100/IR/R

# Selecting a Locomotive (2-digit)

1. Turn knob to select throttle; press SEL/SET.

2. Turn knob to select loco number; press SEL/SET.

# Selecting a Locomotive (4-digit)

1. Hold SEL/SET and turn the desired knob until the ext indicator is lit.

2. Release SEL/SET.

3. Set the first two digits with the left knob and the right digits with the right knob; then press SEL/SET.

# Changing Locomotive Direction

Press L/t for the left throttle or c/R for the right one.

# Releasing a Locomotive or Consist

1. With desired loco active, press SEL/SET.

2. Press MODE/DISP.

# Consisting a Locomotive

1. Set address of TOP loco on Right throttle.

2. Set address of loco to add on Left throttle.

3. Move locomotives into position.

(Make sure both are traveling the same direction!)

4. Press MODE key until MU mode indicator is lit.

5. Press Add (+) key to MU the locomotives.

6. Repeat Steps 2-5 for additional locomotives.

Note: To remove consist press MODE, MODE, (-)

## Activating Functions F0 thru F8

For F0-F4, press FUNC then the desired button.

For F5-F8, press and hold FUNC; then press the desired button.

## Controlling Turnouts

1. Press MODE/DISP until switch indicator is lit.

2. Dial switch address in three left digits.

3. Press L/t for thrown position; c/R for closed.

4. Repeat steps 2-3 for other turnouts.

5. Press MODE/DISP to return to locomotive control.

UT1/UT2

| # LED Status (when selecting loco) |  |
| --- | --- |
| # GREEN | # Loco Acquired |
| # DARK | # Loco In-Use |
| # RED | # SAFE mode - Direction not set to same direction as loco. Flip Direction toggle. |
| # Strobing GREEN | # SAFE mode - Throttle speed does not match loco. Rotate knob direction of strobe. |

# Selecting a Locomotive (2-digit only)

1. Set address on rotary selectors.

2. Press ACQ/DISP key to select loco.

3. See status indications above.

# Acquiring a Dispatched Locomotive

1. Set address to "99" on rotary selectors.

2. Press ACQ/DISP key to select loco.

3. See status indications above.

# Changing Locomotive Direction

Throw the switch right for forward, left for reverse.

# Dispatching a Locomotive or Consist

Press and hold SHIFT; then press DISP.

## Activating Functions F0 thru F6

To toggle F0-F3, press the desired button.

To toggle F4-F6, press and hold SHIFT; then press the desired button.

## Controlling Turnouts (2-digit only)

1. Press RUN/STOP and ACQ at the same time.

2. Select the turnout with the rotary selectors.

3. Press ACQ to throw the turnout.

4. Repeat steps 2-3 for other turnouts.

5. Return selectors to locomotive number; then press RUN/STOP and ACQ at the same time to return to normal operation.

DT300/IR/R

# Selecting a Locomotive (2- or 4-digit)

1. Click knob to select throttle; press SEL.

2. Turn both knobs to select loco number; press SEL.

# Changing Locomotive Direction

Press L/t for the left throttle or c/R for the right one.

Or quickly press the appropriate knob twice.

# Releasing or Dispatching a Locomotive or Consist

1. With desired loco active, press SEL.

2. Press MODE.

# Consisting a Locomotive

1. Set address of TOP loco on Right throttle.

2. Set address of loco to add on Left throttle.

3. Move locomotives into position.

(Make sure both are traveling the same direction!)

4. Press MODE MODE (display shows “Mu”).

5. Press Y/+ to consist the locomotives.

6. Repeat Steps 2-5 for additional locomotives.

Note: To remove loco from consist press MODE, MODE, N/- in step 5.

## Activating Functions F0 thru F8

Press FUNC. Then do any of the following:

- To toggle F0, F1, F3, or F4, press the desired button. 
- F2 does not toggle. Press it as long as you want it on.

- To toggle F5-F8, press and hold FUNC; then press the desired button.

Click or turn a throttle to exit Function mode.

## Controlling Turnouts

1. Press MODE until diplay shows “Sw.”.

2. Dial switch address with either knob.

3. Press L/t for thrown position; c/R for closed.

4. Repeat steps 2-3 for other turnouts.

5. Click a knob to return to locomotive control.